Hardware in the Loop Validation

Hardware-in-the-loop (HIL) simulation is a type of real time simulation. HIL simulation shows how the controller responds to realistic virtual signals. It is also used to check if the model of the physical system (plant) is valid.

The complexity of the plant under control is included in test and development by adding a mathematical representation of all related dynamic systems. These mathematical representations are referred to as the plant simulation.

FIGURE HERE

HIL simulation include emulation of sensors and actuators which act as the interface between the modeled plant and the embedded system under test. The value of each emulated sensor is controlled by the HIL simulation and read by the embedded system under test. Likewise, the embedded system outputs signals that are sent to the actuators on the plant simulation. Changes in the signal sent to the actuator results in changes to variable values in the plant simulation.

Summing up, A HIL simulation is used to test the control technique chose in a given application. With the use of a real-time computer as a virtual representation of the physical system and a real implementation of the controller used, one can test the embedded system before using it in a real hardware with actual components.

In this work, a Typhoon HIL 402 Hardware-in-the-loop system, shown in Fig. 1, is used to test the different control techniques proposed to control the current injected into the grid. Typhoon HIL 402 has the specifications shown in TABLE 11111111.

Fig. 1 Typhoon HIL 402

| HIL402 technical details. |  |
| --- | --- |
| Processor | 4 cores |
| Channels | 16 x Analog inputs (AI) 16 x Analog outputs (AO) 32 x Digital inputs (DI) 32 x Digital outputs (DO) |
| Resolution | 16 bit |
| Analog IO voltage range | ± 10 V |
| Built-in scope | Yes |
| Machine models support | Basic Advanced |
| Connectivity | USB Ethernet |
| Compatibility | HIL DSP Interface HIL Breakout board HIL dSpace Interface HIL uGrid DSP Interface |
| Software | HIL Control Center |

The model used to represent the photovoltaic micro inverter feeding a resistive load with nominal power of 250 W is built in Typhoon HIL Control Center, and is shown in Fig. 2.

Fig. 2. Model of the micro inverter built in Typhoon HIL Control Center

There are two options for controlling the Photovoltaic Micro Inverter switches of Fig. 2: through digital inputs or using an internal modulator. Using the HIL digital inputs means that the micro controller will command the inverter and, thus, the control technique that will be used in the physical prototype can be first tested in the HIL device.

- Validation of the micro inverter model with a resistive load

In order to validate the model that will be used in HIL simulation, the circuit shown in Fig. 2 is compared with the same circuit simulated in PSIM, a computer simulation tool.

There are two options for controlling the Photovoltaic Micro Inverter switches of Fig. 2: through digital inputs or using an internal modulator. Using the HIL digital inputs means that the micro controller will command the inverter and, thus, the control technique that will be used in the physical prototype can be first tested in the HIL device.